Table 5.7. Practice Summary Guide for Pesticide Runoff Losses to Surface Water

| Practice | Potential reduction of Runoff transport |  | Comments |
| --- | --- | --- | --- |
|  | *Strongly adsorbed | Weakly adsorbed |  |
| ## FIELD LOSS REDUCTIONS |  |  |  |
| Lower application rate | M | M | loss reduction should correspond directly to rate reduction |
| Partial substitution | M | M | substituting lower environmental risk pesticides and/or cultural practices will lower pesticide runoff risks |
| Partial treatment | H | M | banding will lower amount of surface area treated and overall application rate; but other cultural practices may be necessary |
| Formulation | L | L | variable based on physical property effects |
| Soil erodibility special restrictions | M | M | restrictions should be targeted to more strongly adsorbed pesticides on highly erodible land and other high runoff areas |
| Soil incorporation | M | H | mechanical incorporation and irrigation reduces amount of pesticide in the surface mixing zone, resulting in reduced soluble losses |
| Application timing | M | H | loss decreases with time between application and storm runoff |
| No-till | H | M | great erosion control; runoff volume much less; high pesticide runoff from surface residue is possible with a large storm event soon after application |
| Conservation tillage | M | M | runoff reduction for first storm after application; enhances infiltration and percolation |
| Subsurface drainage | L | M | improved infiltration and percolation moves water into profile; reduces surface concentration of more soluble pesticides; leached pesticide can be transported to surface waters, but usually at lower concentrations |
| Less soil sealing/compaction | L | M | non-crusted, non-compacted soils allow increased water and pesticide movement into soil profile reducing runoff losses |
| Irrigation management | H | H | irrigation management lowers water use, lowers runoff and erosion; greater infiltration lowers concentration of more soluble pesticides in the soil mixing zone |
| Contour strip cropping | H | H | infiltration within close growing crop strip decreases runoff slightly; sediment and pesticide residues are deposited in the field |
| Crop rotation | H | H | pesticide use can be reduced due to rotational effects on pest complex |
| TRANSPORT REDUCTIONS |  |  |  |
| Terraces/ Detention ponds | H | M | erosion and sediment transport reduction; infiltration and ponding deposits sediment in channels and ponds |
| Constructed wetlands | H | M | deposition of sediment and treatment of runoff |
| Buffer strip | M | M | small, untreated buffer area reduces application field; some infiltration in buffer and sediment deposition |
| Set-back | M | M | more distance to surface water entry points; less inadvertent application directly to water course |
| Vegetated filter strip | H | M | enhanced infiltration; sediment deposited above filter area; filter reduces application area of field |
| Grassed waterway | M | L | concentrated flow restricts treatment; some infiltration and entrapment of pesticides |
| The rough estimates of the likely effects are based on limited research, and professional judgment. It should be possible to predict a more consistent estimate for specific pesticides using a mathematical model for a specific set of soil, climate, and environmental conditions. *Partition coefficient (Kd or Koc) typically > 100 |  |  |  |

# Table 5.9. Practice Summary Guide for Pesticide Leaching Losses to Ground Water

| Practice | Potential reduction of Leaching transport |  | Comments |
| --- | --- | --- | --- |
|  | *Highly Soluble | Slightly Soluble |  |
| ## FIELD LOSS REDUCTIONS |  |  |  |
| lower application rate | H | M | loss reduction should correspond directly to rate reduction |
| partial substitution | H | H | substituting lower environmental risk pesticides and/or cultural practices will lower pesticide leaching risks |
| partial treatment | M | M | reduction of overall application rate will reduce leaching |
| formulation | H | M | less soluble, higher Koc formulations will move slower through soil profile |
| soil leachability special restrictions | H | L | highly leachable soils can be targeted for application and management restriction |
| soil incorporation | M | L | moves pesticide into soil profile, reduces macropore flow |
| application timing | H | L | application before expected precipitation event can drive pesticide lower in profile pre-emergent, thus encourages leaching; post-emergent susceptible to leaching than pre-planted |
| no-till | L | L | macropores created in soil profile speeds leaching, higher water infiltration as percolation |
| conservation tillage | M | L | runoff reduction enhances infiltration and percolation, reduces macropores |
| subsurface drainage | M | L | improved infiltration percolates more water, leached pesticide can be transported to surface waters |
| less soil sealing compaction | L | L | non-crusted, non-compacted soils allow increased pesticide movement into soil profile |
| irrigation management | H | L | management lowers water use, lowers leaching losses |
| Contour strip cropping | L | L | Infiltration within close growing crop strip increases leaching slightly |
| crop rotation | M | L | rotating pesticide with crop reduces any one chemical to carry over and build up in soil profile |
| TRANSPORT REDUCTIONS |  |  |  |
| Terraces Detention ponds | L | M | some enhanced leaching below terrace channel and bottom of pond |
| Constructed wetlands | L | L | leaching can be expected below wetlands |
| Buffer strip | L | L | small, untreated buffer area reduces application field; enhanced infiltration in buffer; lengthens flow path through soil |
| Set-back | M | L | more distance to ground water entry point like wells and sink holes |
| Vegetated filter strip | L | L | enhanced infiltration; filter reduces application area of field |
| Grassed waterway | L | L | concentrated flow restricts infiltration |
| The rough estimates of the likely effects are based on limited research and professional judgment. It should be possible to predict a more consistent estimate for specific pesticides using a mathematical model for a specific set of soil, climate, and environmental conditions. * Partition coefficient (Kd or Koc) typically <300 and solubility >30 ppm for highly soluble pesticides. |  |  |  |