10 Mile Lake Water Management Project

Appendix D-4: PIFR Technical Analysis

Icelandic State Park

Renwick Dam

Prepared for:

Barnes County Water Resource District

PO Box 306

Valley City, ND  58072

Prepared by:

Natural Resources Conservation Service

North Dakota State Office

220 E Rosser Ave, Box 1458

Bismarck, ND  58502-1458

August 22, 2024

Contents

1- Estimation of Wetland Extents and Types	3

2- Mitigation Cost Estimates	4

3- Preliminary Engineering Alternatives	5

4- Historic Crop Types, Yield Projections	7

5- Economic Benefits Analysis	8

6 – Benefit Cost Summary	10

7 - References	10

8 – List of Preparers (PIFR and/or PIFR Technical Appendix)	10

Table 1 - Alternative 1 Wetlands Potentially Eligible to Drain	4

Table 2 - Alternative 2 Wetlands Potentially Eligible to Drain	4

Table 3- Mitigation Requirements	5

Table 4- Construction Costs	6

Table 5- Total implementation Costs, Alternative 1	6

Table 6- Total Implementation Costs, Alternative 2	6

Table 7- Operation and Maintenance Costs	7

Table 8- Historic Crop Data	7

Table 9- Yield Estimates	7

Table 10- Crop Revenues and Costs	8

Table 11- Production Benefits Due to Newly Created Cropland	8

Table 12- Production Benefits Due to Tile Drainage Installation	8

Table 13- Benefits Due to Reduced Road Maintenance	9

Table 14- Alternative Net Present Value, Benefit-to-Cost Ratio Summary	9

Figure 1 - 2021 Crop Data	13

Figure 2 - 2022 Crop Data	14

Figure 3 - 2023 Crop Data	15

Figure 4 - Wetland 16, Potential Mitigation Site 	16

Figure 5 - Wetland 18 Potential Mitigation Site	17

This technical appendix report is provided to document the field work, wetland mitigation analysis, preliminary design, and preliminary economic analysis work completed to assess feasibility of improvements a water management project on 10 Mile Lake as required for the Preliminary Investigation of Feasibility.

# 1- Estimation of Wetland Extents and Types

The project area for the 10 Mile Preliminary Investigation Findings Report (PIFR) was chosen based on proximity of land to 10 Mile Lake and the proposed outlet channel.  The fields immediately surrounding the lake were chosen because the newly exposed cropland (former lake bottom) would likely need to have subsurface drainage installed to help control inevitable salinity issues.  The potential profitible installation of subsurface drainage in the rest of those fields surrounding the lake was positive.  A buffer of approximately two miles was used along the proposed outlet channel was used as the remainder of the project area.  The topography in this area is conducive to tile drainage into the proposed channel.  Not all wetlands within the project area are eligible for drainage.  Many of the wetlands in the area are protected from drainage either through easements or fee title ownership by government agencies.  The wetlands that could potentially eligible for drainage under each alternative are listed in the tables below and were assumed to all be drained under Alternative 1 for increased crop production in the watershed.

Wetlands play an important role in in the water cycle, reducing flooding, providing filtration and recharge for aquafers, in addition to many other benefits.  Because of the important role wetlands play in the nation’s ecosystems, the U.S. government has put restrictions on the use of federal funds for activities that would negatively impact wetlands.  Executive Order 11990 states: Each agency shall provide leadership and shall take action to minimize the destruction, loss or degradation of wetlands, and to preserve and enhance the natural and beneficial values of wetlands in carrying out the agency's responsibilities for (2) providing Federally undertaken, financed, or assisted construction and improvements; and (3) conducting Federal activities and programs affecting land use, including but not limited to water and related land resources planning, regulating, and licensing activities (E.O. 11990, Sec.1(a)).  Which basically says the federal government can not fund projects that will adversely affect wetlands.  The work around for this situation is through mitigation.  Table 1 and Table 2 below show the wetland acreages and types of wetlands eligible for drainage, and therefore mitigation.  Wetlands are defined as those areas that are inundated by surface or ground water with a frequency sufficient to support and under normal circumstances does or would support a prevalence of vegetative or aquatic life that requires saturated or seasonally saturated soil conditions for growth and reproduction. Wetlands generally include swamps, marshes, bogs, and similar areas such as sloughs, potholes, wet meadows, river overflows, mud flats, and natural ponds.  The question then arises regarding the need to mitigate the portion of 10 Mile Lake classified as deepwater habitat, because it does not meet the vegetative requirements of the wetland definition.  The deepwater habitat needs to be mitigated for because a large portion of 10 Mile Lake is classified as lacustrine fringe habitat, which does meet the definition of a wetland.  Without deepwater habitat, lacustrine fringe habitat cannot exist.  Typically, a hydrogeomorphic model (HGM) would be used to determine the mitigation extent necessary to offset the conversion activity, however, no HGM for deepwater habitat or lacustrine fringe habitat exists for the northern great plains.  Instead, during an interagency meeting,  it was decided that using a 2:1 ratio approved by NDIRT, and used often during the construction of mitigation banks, would be used to calculate mitigation needs for this PIFR.

Table 1 - Alternative 1 Wetlands Potentially Eligible to Drain

| Wetland Code | Wetland Type | Acres | Count |
| --- | --- | --- | --- |
| L2AB, G, d, x | Lake | 1523.5 | 14 |
| PABF, x | Freshwater Pond | 24.5 | 34 |
| PEM1A, x | Freshwater Emergent | 255.6 | 278 |
| PEM1C, d, x | Freshwater Emergent | 608.3 | 140 |
| PEM1F | Freshwater Emergent | 0.9 | 2 |
| Pf | Other (farmed) | 337.4 | 897 |
| PFO1A | Freshwater Forested | 1.8 | 2 |
| PSS1A | Freshwater Shrub | 0.1 | 1 |
| R4SBA, x | Riverine | 1.5 | 3 |
| R4SBC, x | Riverine | 3.8 | 6 |
|  | Total | 2607.1 | 1,377 |

Table 2 - Alternative 2 Wetlands Potentially Eligible to Drain

| Wetland Code | Wetland Type | Acres | Count |
| --- | --- | --- | --- |
| L2AB, G, d, x | Lake | 1523.5 | 14 |
| PABF | Freshwater Pond | 1.4 | 3 |
| PEM1A, d | Freshwater Emergent | 20.1 | 36 |
| PEM1C, d | Freshwater Emergent | 85.9 | 35 |
| Pf | Other (farmed) | 48.2 | 194 |
|  | Total | 1679.1 | 282 |

# 2- Mitigation Cost Estimates

In order to complete a cursory economic analysis for project alternatives, it was necessary to develop cost estimates for the wetland and deepwater habitat mitigation requirements outlined above.  Compensatory wetland mitigation in North Dakota is coordinated through the North Dakota Interagency Review Team (NDIRT) made up of NRCS, USACE, EPA, and USFWS which has published technical guidelines outlining requirements for mitigation bank sponsors.  The 10 Mile Lake watershed lies within the Red River wetland mitigation service area. Mitigation credits may be purchased by project developers through existing approved banks, however currently in the Red River there 175 acres of mitigation credits available for purchase, for Clean Water Act wetland mitigation, at an average price of $63,000/acre currently.  Clearly this is only a small fraction of what would be necessary for the 10 Mile Lake project and there is no availability of credits for lacustrine fringe or deepwater habitat mitigation. The practical and cost-effective approach would be for the Barnes WRD to develop their own mitigation sites in partnership with an experienced mitigation bank entity operating in ND.  As a part of the PL-566 project, Barnes WRD would purchase land rights, complete construction of restoration/creation projects under the NDIRT guidelines and turn over long term management of the mitigation bank properties to the partner.  A multitude of sites spread across the Red River watershed would likely be necessary, although sites in Barnes County near the project would be preferred.

Recognizing that wetland mitigation would be a major cost to the project, NRCS invited Barnes WRD to identify any cost-effective mitigation sites in the county.  Barnes WRD hired Houston Engineering to assist with this.  HEI and NRCS looked at 4 potential sites.  The sites (labeled 16, 17, 18, and 22) are all wetlands with pre-existing drainage, however, none are fully converted.  NRCS and HEI completed wetland delineations according to USACOE wetland delineation procedures.  Of the 4 sites, 2 have the potential to generate wetland credits.  Wetland 16 is located within the SE¼ of Sec. 35 T142N R61W and the N½ of Sec. 2 T141N R61W, Barnes County ND.  It has the potential to generate 23.2 credits.   Wetland 18 is located within the NE ¼ Sec. 35 T142N R61W.  It has the potential to generate 13.7 credits.   Total potential number of credits between the two wetlands is 36.9.  These credits would be suitable for depressional wetlands, not lacustrine fringe or deepwater habitat.  (See Figures 4 and 5) Unfortunately, 36.9 is only a fraction of the credits required for depressional wetlands, let alone the credits required for lacustrine fringe and deepwater habitat.

Given that the goal of the PIFR was to simply complete a feasibility level economic analysis, NRCS intentionally utilized a very optimistic cost estimate of $20,000/acre for mitigation of depressional wetlands and assumed these could be developed in areas currently not being farmed.  Note that if mitigation were to occur on cropland, the economic loss of taking that land out of production would further reduce the overall economic benefits of this project.   Development of a site to mitigation the 1,254 acres of deepwater habitat and 539 acres of lacustrine fringe wetlands would logically involve construction of a dam.  A dam on the Upper Maple River was constructed in Steele County in 2015 at a total cost of $9.2 million, including land rights; adjusted to 2024 that would be $11.8 million.  The dam has a maximum height of 30 feet and impounds 5,205 acre-feet of water to the auxiliary spillway crest.  It was constructed as a dry dam; however, construction costs would be similar if a principal spillway design to create a permanent pool would have been included.  The stage-storage curve for the dam indicates that this project could have generated 387 acres of deepwater habitat, and 338 acres of lacustrine fringe wetlands and mitigation costs were estimated accordingly. Note that this is again a generous estimate; in reality, construction of a dam with a permanent pool would likely require extensive mitigation of riverine and depressional wetlands that would be flooded by the permanent pool level.  In addition to those potentially high wetland mitigation costs, any cropland taken out of production by the new dam would also have to be accounted for in the PL-566 economic analysis. Table 3 provides a summary of the mitigation cost assumptions utilized for this analysis, with the recognition that this is an unlikely “best case” scenario from an economic standpoint.

Table 3- Mitigation Requirements

| Mitigation Type | Mitigation Development Assumption | Alt 1 | Alt 1 Mitigation Cost | Alt 2 | Alt 2 Mitigation Cost |
| --- | --- | --- | --- | --- | --- |
| Deepwater Habitat | $30,332/ac | 1,254 ac | $38,036,193 | 1,254 ac | $38,036,193 |
| Lacustrine Fringe Wetlands | $34,726/ac | 539 ac | $18,717,089 | 539 ac | $18,717,089 |
| Depressional Wetlands | $20,000/ac | 1,612 ac | $32,240,000 | 156 ac | $3,120,000 |
| Total |  | 3,405 ac | $88,993,282 | 1,949 ac | $59,873,282 |

# 3- Preliminary Engineering Alternatives

As noted in the PIFR, a preliminary design had already been developed for a control structure and outlet channel designed to lower 10 Mile Lake by 4 feet by Houston Engineering (see attachment).  NRCS laid out the alternative channel location shown in Appendix 2, Exhibits 10 and 11 to minimize wetland impacts, as required by E.O. 11990.  The control structure would likely be a corrugated metal riser with incorporated slide gate. Houston Engineering provided an approximate construction cost estimate for the control structure and channel, in 2024 dollars, as listed below based on proportioning the length of the previous design to the new alignment.

To evaluate both a minimum and maximum approach to installing subsurface drainage in cropland, to look at benefit-to-cost ratios, two alternatives were developed.  Alternative 1 maximizes cropland drainage along the outlet channel, to the extent possible given topography and existing conservation easements, for the purpose of maximizing benefits. Alternative 2 includes only cropland drainage immediately adjacent to 10-Mile Lake, for the purpose of minimizing costs.  In reality, there would be a full range of potential scenarios between the two. Costs for cropland drainage were derived from the Ellingson Tile report (see attachment) average for 3/8” drainage coefficient for tiling.

Table 4- Construction Costs

| Item | Quantity | Unit | Rate | Construction Cost |
| --- | --- | --- | --- | --- |
| Furnish and install 36” pipe (incl. road crossings and field accesses) | 6,040 | LF | $125 | $755,000 |
| Drainage channel excavation (incl, topsoil, seeding, etc.) | 36,300 | LF | $66 | $2,395,000 |
| Furnish and install water control structure | 1 | LS | $50,000 | $50,000 |
| Erosion control | 1 | LS | $50,000 | $50,000 |
| Mobilization, overhead, traffic control, contractor QC | 1 | LS | $150,000 | $150,000 |
| Subtotal – control structure and channel |  |  |  | $3,400,800 |
| Alternative 1 |  |  |  |  |
| Subsurface tile installation | 13,329 | AC | $1,140 | $15,195,060 |
| Total |  |  |  | $18,595,860 |
| Alternative 2 |  |  |  |  |
| Subsurface tile installation | 539 | AC | $1,140 | $2,431,620 |
| Total |  |  |  | $5,832,420 |

Total implementation costs, including engineering, administration, land rights, and utilities were estimated as follows and percentages applied up to the maximum federal share allowed by the NRCS policy.  Note that it is assumed that the WRD would develop contracts with private owners of fields to be tiled (with the private owner funding the non-federal share of tiling) and that construction easements for that work would be at no charge to the project.

Table 5- Total implementation Costs, Alternative 1

| Item | Total Cost | NRCS | Local/State |
| --- | --- | --- | --- |
| Engineering Design | $816,192 | $816,192 | $0 |
| Construction Engineering | $544,128 | $544,128 | $0 |
| Construction | $18,595,860 | $13,946,895 | $4,648,965 |
| Sponsor Legal/Contract Admin Costs | $85,000 | $0 | $85,000 |
| Land Rights (not including mitigation) | $1,125,000 | $0 | $1,125,000 |
| Wetland / Deepwater Habitat Mitigation (construction + land rights) | $88,993,282 | $44,496,641 | $44,496,641 |
| Utility Relocations | $215,000 | $0 | $215,000 |
| Total | $110,374,462 | $59,803,856 | $50,570,606 |

Table 6- Total Implementation Costs, Alternative 2

| Item | Total Cost | NRCS | Local/State |
| --- | --- | --- | --- |
| Engineering Design | $408,096 | $408,096 | $0 |
| Construction Engineering | $272,064 | $272,064 | $0 |
| Construction | $5,832,420 | $4,374,315 | $1,458,105 |
| Sponsor Legal/Contract Admin Costs | $85,000 | $0 | $85,000 |
| Land Rights (not including mitigation) | $1,125,000 | $0 | $1,125,00 |
| Wetland / Deepwater Habitat Mitigation (construction + land rights) | $59,873,282 | $29,936,641 | $29,936,641 |
| Utility Relocations | $215,000 | $0 | $215,000 |
| Total | $67,810.862 | $34,991,116 | $32,819,746 |

An estimate of $5,000 was assumed for annual operation and maintenance costs for the control structure and channel and $3/acre was assumed for tile drainage systems in crop fields, which would including energy costs for pumping.   The O&M for wetland mitigation sites was assumed to be included in the per acre rates.

Table 7- Operation and Maintenance Costs

| Item | Alt 1 | Alt 2 |
| --- | --- | --- |
| Annual O & M Costs Channel/Outlet Structure | $5,000 | $5,000 |
| Annual O&M Costs Tile Drainage | $39,987 | $6,399 |
| Total | $44,987 | $11,399 |

# 4- Historic Crop Types, Yield Projections

The USDA National Agricultural Statistics Services (NASS) Cropland Data Layer (CDL) provides remote sensing derived raster data for the U.S. on an annual basis.  In North Dakota, this source is generally considered to be the most reliable data source for crop type estimates at a watershed scale.  Data was summarized for the watershed for 2021, 2022, and 2023 as shown in Figures 1, 2, and 3 which are summarized in Table 8.

Table 8- Historic Crop Data

| Crop | 2021 (acres) | 2022 (acres) | 2023 (acres) | Avg. (acres) | % of Total |
| --- | --- | --- | --- | --- | --- |
| Corn | 9,289 | 2,210 | 4,537 | 5,345 | 24% |
| Soybeans | 10,841 | 10,043 | 6,854 | 9,246 | 41% |
| Small Grains (Wheat, Barley, Rye) | 4,021 | 4,000 | 8,940 | 5,654 | 25% |
| Dry Beans | 80 | 640 | 0 | 240 | 1% |
| Alfalfa | 80 | 80 | 80 | 80 | 0% |
| Grass/Pasture | 2,055 | 2,055 | 2,055 | 2,055 | 9% |
| Total | 26,366 | 19,028 | 22,466 | 22,920 |  |

The proposed project alternatives would be targeted to improving production on row crops, therefore only the percentage of row crops in the watershed was relevant to the analysis.  Predictive equations for yield improvements due to drainage are not available for dry beans, barley, and rye and spring wheat is the predominant small grain crop grown in Barnes County. Therefore, the  percentages were adjusted for economic analysis as listed in Table 9.  Crop yields, with and without subsurface tile, were taken from the Ellingson Tile Drainage Assessment (see attachment) prepared for the project area.  Note that the yields without tile reported in the Ellingson report match to the NDSU

Table 9- Yield Estimates

| Crop | % of Total | Average Undrained Cropland Yield (bu/ac) | Average Drained Cropland Yield (bu/ac) |
| --- | --- | --- | --- |
| Corn | 26% | 143.4 | 178.1 |
| Soybeans | 46% | 37.5 | 42.9 |
| Spring Wheat | 28% | 59.9 | 69.8 |

# 5- Economic Benefits Analysis

The analysis relies on the procedures and guidance provided in the Economic and Environmental Principles and Guidelines for Water and Related Land Resources Implementation Studies (P&G), the Principles and Requirements for Federal Investments in Water Resources (PR&G), and the National Resources Economics Handbook (NREH) part 611. The analysis uses the Fiscal Year (FY) 2024 federal discount rate for water resources projects of 2.75%.

Crop Production Benefits

The National Resource Economics Handbook 611.0102(b)(1) directs use of current normalized prices for economic evaluation of watershed projects. The state-level normalized prices as provided by the Economic Research Service (ERS) for 2023 were utilized for revenues.  For land newly brought into production due to the project, the full annual direct production costs from the NDSU 2024 Crop Budgets for South East North Dakota was utilized.  For the additional yield on tiled land, it was assumed that the only production costs that would increase would be drying and hauling costs.

Table 10- Crop Revenues and Costs

| Crop | Price | Full Production Costs | Drying and Hauling Costs |
| --- | --- | --- | --- |
| Corn | $4.00/bu | $422.30/ac | $0.17/bu |
| Soybeans | $9.50/bu | $320.33/ac | $0.20/bu |
| Spring Wheat | $5.99/bu | $229.88/ac | $0.17/bu |

The resulting annual crop production benefits due to newly created cropland under each alternative are outlined below.

Table 11- Production Benefits Due to Newly Created Cropland

| Crop | Alt 1 New Cropland (ac) | Alt 1 Net Revenue Increase | Alt 2 New Cropland (ac) | Alt 2 Net Revenue Increase |
| --- | --- | --- | --- | --- |
| Corn | 153 | $44,425 | 90 | $26,043 |
| Soybeans | 265 | $23,104 | 155 | $13,543 |
| Spring Wheat | 162 | $30,487 | 95 | $17,872 |
| Total | 580 | $98,016 | 340 | $57,458 |

The resulting annual crop production benefits due to installation of drain tile on existing cropland under each alternative are outlined below.

Table 12- Production Benefits Due to Tile Drainage Installation

| Crop | Alt 1 Drained Cropland (ac) | Alt 1 Net Revenue Increase | Alt 2 Drained Cropland (ac) | Alt 2 Net Revenue Increase |
| --- | --- | --- | --- | --- |
| Corn | 3,519 | $471,625 | 563 | $75,473 |
| Soybeans | 6,087 | $302,880 | 974 | $48,469 |
| Spring Wheat | 3,722 | $213,821 | 596 | $34,217 |
| Total | 13,329 | $988,326 | 2,133 | $158,159 |

Road Maintenance Benefits

NRCS contacted the township boards responsible for road maintenance and repairs around the lake and received the following information on road maintenance costs due to high water that would be avoided with construction of this project, over the last 5 years.

Table 13- Benefits Due to Reduced Road Maintenance

| Year | Township | High Water Road Repair Costs |
| --- | --- | --- |
| 2019 | Laketown | $33,184 |
| 2020 | Laketown | $133,831 |
| 2021 | Edna | $102,265 |
| 2022 | ------ | that------ |
| 2023 | Laketown | $ |
| Total |  | $988,326 |
| Average Annual Costs |  | $61,243 |

City of Dazey Municipal Benefits – Buildings, Streets, Sewage Lagoons

NRCS contacted both the former and current mayors of Dazey to request information on expenditures related to high water tables.  The city has spent substantial funds on lift stations, ditch clean out, street repairs, and work on the sewage lagoons due to high groundwater but could not easily put together historical data other than a recent expenditure of $ 6,817.  An estimate of $20,000 a year was utilized, as an intentionally liberal value in computing the potential benefits of the project in terms of avoided costs to Dazey.

Residential Benefits – Sump Pump O&M, Basement Refinishing, Mold Remediation

NRCS requested information from the Barnes WRD, Moore Engineering, and directly asked a resident but was unable to glean any specific information on costs related to high ground water control in basements, although it is generally known to be a persistent issue for many of the 81 homes in the area (55 homes within Dazey and 26 rural homes).  An estimate of $20,000 a year was utilized, as an intentionally liberal value in computing the potential benefits of the project in terms of avoided costs to private homeowners.

# 6 – Benefit Cost Summary

The net present values and benefit to cost ratio of each alternative are listed in Table 14.

Discount rate = 2.75%

Construction Years = 2024-37

Project Life = 50 Years

Construction Period + Project Life = 50 Years

Table 14- Alternative Net Present Value, Benefit-to-Cost Ratio Summary

| Item | Alternative 1 Net Present Value | Alternative 2 Net Present Value |
| --- | --- | --- |
| Costs |  |  |
| Implementation Costs | $ 104,571,051 | $64,245,415 |
| Operation and Maintenance Costs | $ 1,119,592 | $283,687 |
| Financing Costs | $ 5,911,746 | $3,632,005 |
| Total Costs | $ 111,602,389 | $68,161,107 |
| Benefits |  |  |
| Revenue from New Cropland Under Production | $ 2,439,323 | $1,429,948 |
| Revenue from Tiling Existing Cropland | $ 15,496,875 | $3,090,681 |
| Reduced Road Maintenance | $ 1,524,160 | $ 1,524,160 |
| Reduced Residential Groundwater Costs | $ 497,740 | $ 497,740 |
| Reduced Municipal Groundwater Costs | $ 497,740 | $ 497,740 |
| Total Benefits | $ 20,455,839 | $7,040,270 |
| Benefit/Cost | 0.18 | 0.10 |
| Net Benefits | ($91,146,550) | (61,120,837) |

# 7 - References

Economic Research Service (ERS), U.S. Department of Agriculture (USDA). “Normalized Prices”. September 30, 2023. https://www.ers.usda.gov/data-products/normalized-prices/

Ellingson Water Management, 2024. 10 Mile Lake Drainage Assessment.

Haugen, 2024. North Dakota State University (NDSU) Extension. Farm Management Planning Guide EC 1659 Projected 2024 Crop Budgets: South East North Dakota. https://www.ndsu.edu/agriculture/ag-hub/ag-topics/farm-managemetn/crop-economics/projected-crop-budgets.

North Dakota State University. 2020 North Dakota Custom Rates Part II – Late Season Operations, December 2020. https://www.ndsu.edu/agriculture/ag-hub/ag-topics/farm-management/2020-north-dakota-custom-rates.

NRCS National Bulletin NB 200-24-1 ECN – Discount Rate for Fiscal Year 2023 Water Resources Planning and Evaluation. November 1, 2023.

# 8 – List of Preparers (PIFR and/or PIFR Technical Appendix)

Christi Fisher, P.E. State Conservation Engineer

Jon Petersen, P.E. Hydraulic Engineer

John Bauer, Watershed Planner

Rita Sveen, Watershed Planner

Dana Whippo, Economist

Richard Webb, State Resource Conservationist

Curt Bradbury, Biologist

Technical assistance on field wetland extents, preliminary design, and construction cost estimates from Houston Engineering, Inc:

Mike Opat, P.E.  Senior Civil Engineer

Mark Aanenson,  Senior Environmental Scientist

Technical assistance for mitigation needs and requirements:

J. Reinisch – USFWS

T. Erhardt – USACOE

B. Soiseth – USACOE

M. Linsay – NDDWR

J. Martin - NDDWR

Attachments:

Ellingson Water Management, 2024.  Drainage Assessment 10 Mile Lake

Moore Engineering, 2021.  Preliminary Plans 10 Mile Outlet

Figure 1 - 2021 Crop Data

Figure 2 - 2022 Crop Data

Figure 3 - 2023 Crop Data

Figure 4 - Wetland 16, Potential Mitigation Site

Figure 5 - Wetland 18 Potential Mitigation Site